R-LINK USER MANUAL

Latest version  1.0.7.1

Compiled by R&D Dipes Team

10/2024

Contents

Chapter 1 – Introduction	5

R-Link overview	5

System requirements	5

Chapter 2 - Getting Started	6

Installing R-Link	6

Updating R-Link	8

Uninstalling R-Link	8

Chapter 3 – License and Login	9

License to use the software	9

Log in to use the software	10

Chapter 4 – Essential Equipment	11

List of essential system equipment	11

Chapter 5 – Settings	12

Settings menu	12

System settings	13

Camera settings	14

Printer settings	17

PLC Settings	18

Scanner settings	19

Chapter 6 - Job Creation	20

Create a new Job	20

Compare type	21

Database path	22

Comparative data format	22

Template name	26

Save Job	26

Chapter 7 – Job Selection	28

Select job tab	28

Job details	28

Buttons to manipulate Job	29

Job Classification	29

Chapter 8 – Job execution page	30

Job details	30

Image display box	31

Verification results	31

Verification progress	32

Statistics of POD data	32

Database View	33

Job logs	35

Job Logs Detail	36

Chapter 9 – Users management	38

Users Management	38

Users management functions	39

User management operations	40

Chapter 10 – Program history	43

Log datetime filter	45

Chapter 11 – Export reports for Job	46

Export print data report	46

Export verification data report	47

Chapter 12 – Another functions	48

Return to Job selection page	48

Status bar and other information	48

Chapter 13 – Rynan printer settings	49

Check all printer settings function	49

Notification of printer parameters not enabled	51

Printer connection settings	51

Design a template	52

Chapter 14 – Cognex Dataman camera settings	54

Software required	54

Connect to camera	54

Optimize Image	56

Identify barcode type	56

Trigger type	57

Output settings	58

Chapter 15 – Cognex In-Sight Vision camera settings	60

# Chapter 1 – Introduction

## R-Link overview

R-Link is a barcode verification software used to ensure the reliability of data in printing systems for products such as goods, packaging, etc. The R-Link software is suitable for use in industrial printing systems and is optimized for Rynan R Series printers to perform data printing processes. It supports connection with cameras such as Cognex, Keyence, etc., to capture images of printed data. Finally, R-Link verifies the results and performs the process of sorting and marking errors. Additionally, R-Link can operate in a verification-only mode without coordinating data printing, making it compatible with any printer system. In this mode, R-Link will only read the printed data, compare, and then classify or mark errors.

R-Link is designed to operate in high-speed data printing systems and is continuously upgraded with new features and actively supported by a dedicated team.

## System requirements

- Operating System: Windows 10 Professional (64-bit), version 20H2 or later; Windows 11 (64-bit), version 23H2 is recommended

- Screen resolution: Recommended 1920x1080

- Tested CPU: Intel® i5 3th, Intel® 12th Gen or newer, or AMD Ryzen™ 4000 Series or newer

- RAM: 8GB or more

- Local Storage: SSD with 1GB of available disk space

- Internet: Not required

- Additional system requirements: Microsoft .NET 4.8 or later (included)

- Recommended network configuration: Cat5e/Cat6e, Gigabit Ethernet (1 Gbps) or higher, PCIe x4, RJ45/Optical Connector.

# Chapter 2 - Getting Started

## Installing R-Link

Unzip the installation file (.zip) and run the (.exe) format file

On startup settings interface:

- Select “Create a desktop shortcut” to display the software icon on the desktop, click Next.

- Click the Install button to start the software installation process

- Once the installation is complete, select Finish to start the software right away (if you choose the Launch R-Link option)

## Updating R-Link

To update the R-Link software, a new version software installation file is required and perform the installation normally (no need to uninstall the previous version).

## Uninstalling R-Link

To uninstall the R-Link software you need to go to the Control Panel and select Uninstall a program.

Select the R-Link software and uninstall it.

# Chapter 3 – License and Login

## License to use the software

R-Link uses a USB Secure Dongle for activation. Users will receive a notification if they do not have a license to use the software.

Plug the R-Link keyed USB Dongle Key into the computer to use the R-Link software

## Log in to use the software

Default login account to use the software

- Username: admin

- Password: 123456

# Chapter 4 – Essential Equipment

## List of essential system equipment

| Devices name | Functions | Minimum quantity |
| --- | --- | --- |
| Personal Computer | R-Link installation environment | 1 |
| Printer (accessory) | Print barcodes | 1 |
| Camera (accessory) | Read barcode data | 1 |
| Controller system | Use PLC to classify and mark errors | 1 |
| Sensor 1 | Camera trigger sensor | 1 |
| Sensor 2 | Printer trigger sensor | 1 |
| Encoder 1 | Printer encoder | 1 |
| Encoder 2 | PLC encoder | 1 |
| Encoder 3 | Mark errors printer | 1 |

Note: This is a list of the minimum basic equipment required to set up a barcode verification system.

# Chapter 5 – Settings

## Settings menu

After successfully logging in, the interface below will be displayed first

To enter the settings section, you need to select the Settings section in the vertical menu bar

Settings interface

## System settings

System settings include:

Language: Support English and Vietnamese

Job format: The name format for Job includes Datetime format and File name

Completion condition: include Total checked & Total passed

- Total checked: The running process will stop when the total amount of checked data is reached, For example: Database has 1000 barcodes, it will stop if the camera captures exactly 1000 of those codes.

- Total passed: The process will stop when the required number of databases have been verified and passed. For example, if there are 1000 barcodes, the process will only stop once all 1000 codes have been checked and passed.

Output signal: The camera output controlled by R-Link toggles between Enable and Disable each time an error code is detected during verification.

Image export: failed image export function to make image log

Data sending method for Verify and Print mode: Customize the data sent to the printer in Verify and Print mode

- Basic: The camera reads and compares data. If the comparison is valid, it sends the data to the printer. If invalid, it sends a user-entered string to the printer (default is 'Failure').

- Compare: The camera also reads and compares data. If correct, it can send fields in the database in a customized way or send all of them. If invalid, it sends a user-entered string to the printer (default is 'Failure').

Users can also change the character string sent to the printer when the comparison is invalid.

## Camera settings

Camera setting interface

Camera settings include:

Type: camera types

- Cameras supported: Cognex Dataman, Cognex In-Sight Vision

DM Series (Cognex Dataman camera)

- IP Address

- Information about cameras:  Serial number, Model, Dataman read mode

Dataman read mode include:

- Basic Read (Image+Code): used for a single camera to retrieve images and code content

- Multi-Sync Read with TCP/IP (Code): Using multiple dataman cameras in combination to produce a single result, it would be impossible to obtain images. This mode requires using the Multi-Sync function on the camera and executing a custom script on Dataman Setup Tool

How to use Multi-Sync Read with TCP/IP (Code)

* Notes: This mode will be performed by R-Link technicians based on customer needs. Customers are not encouraged to configure themselves

- Open Dataman Setup Tool and create new group

Documents :

DataMan Multi-Reader Sync Configuration: https://support.cognex.com/docs/dmst_2421/web/EN/Fixed_Mount_Manual/Content/Topics/FMREF/DataManMasterSlaveConfig/DataManMultiReaderSync.htm

MRS Tutorial 02 - Basic Setup

https://www.cognex.com/videos/barcode-readers/mrs-tutorial-02-basic-setup

Data formating script:

Replace the slave variable values ​​with the camera models

Example: var slave = [“camera slave model 1”,” camera slave model 2”,…]

IS Series (Cognex Insight-Vision camera)

Use an In-Sight vision camera

- IP Address

- Information about cameras:  Serial number, Model

- Image Resolution: resolution of the image captured from the camera

Camera Insight-Vision object settings

To install this part, you need to connect the camera on Insight-Vision Suite software

ObjectName: is the name of the object trained on Insight-Vision Suite software

- ID Read: read barcodes

- Character Read: Read characters, strings, numbers

- Single Job file name: name of the job file created on Insight-Vision Suite

IS Series Dual (Combines results from two In-Sight Vision cameras)

Same as IS Series but with an additional slave camera

## Printer settings

Printer settings interface

POD Protocol

- Printer connection parameters: IP Address, Port, Printer remote port

- Advanced printer settings: Use the printer configuration via the web interface

- Acceleration POD Send: Increase CPU usage for data transmission from R-Link to the printer (Limit abuse if you don't have to run at high speed)

Successful connection result: the connection icon turns blue

## PLC Settings

PLC settings interface

The system will use a PLC to control error marking, connected through the IP address of the PLC. This will create a sensor control mechanism that marks errors.

- Connection parameters: IP Address, Port

- Response content: Feedback data from PLC, Clear response button to clear feedback data

- Sensor parameters: Parameters for error marking at the PLC

Delay sensor (mm), Disable sensor (mm), Pulse Encoder (ppr), Encoder Diameter (mm), GAP, Error Condition (Unit)

## Scanner settings

Scanner settings interface

Connect to handheld barcode scanner via serial protocol

Steps to connect to a handheld Barcode scanner

- Configure Barcode Scanner to serial protocol

- Enter the corresponding COM port name

Open Device Manager on your computer to find the COM name of the barcode scanner

# Chapter 6 - Job Creation

## Create a new Job

To start a process, you must first create a series of information about the running process, this process creates a file called Job, a Job will include:

Job name: The name for Job can be created with the click of a button

## Compare type

+ Can read: non-printer mode (change the settings in the Printer series section to Standalone). If the camera returns a non-empty result, the result is valid; otherwise, if the result is empty, it is invalid.

+ Static text: non-printer mode (change the settings in the Printer series section to Standalone). If the camera returns a result that matches the static text input, the result is valid; otherwise, it is invalid.

Database: use a database file from the user to compare with camera results. Can be used with RYNAN R-Series printer or Standalone

Job type:

- After production: A mode that prints first, then takes an image of the data and compares with comparative data format to mark errors. This mode requires the use of a database for comparison

- On production: This mode prints first, then takes an image of the data and compares it with the comparative data format. If the result is valid, it prints the next data. Otherwise, it reprints the previous invalid data. The purpose is to ensure the correct printing order of the data series.

- Verify and print: This mode takes an image of the data first and then compares this data with the comparative data format. The result will be sending data according to the settings in the "Data sending method for Verify and Print mode" section, see System settings section.

## Database path

database path in (.csv) format that the user selects

## Comparative data format

select the comparison format based on the fields in the user's database.

List of fields from the database: List of fields available in the database

Comparative data format: format that the user has selected

Navigation buttons:

Select desired design school

Cancel selected design field

Move selected field up

Move selected field down

Clear all selected fields

Text field content: optional character or text that the user enters

Format of the verification field: formatted fields for syntax preview purposes

Preview comparison data: Preview formatted data as actual values

Database view : view original database format

## Template name

Select the template to print, this template is listed and uploaded from the printer

Use the button  to refresh the template list

## Save Job

After entering the necessary options, finally use the Save button to create a new Job.

After successfully saving, a confirmation dialog box appears to Start the process immediately

Select Yes if you want to move to the process start page, Select No if you want to do this at another time.

# Chapter 7 – Job Selection

## Select job tab

After creating a Job, the list of Jobs will be listed in the Select job Tab. Each Job will have a .rvis extension, which will be saved on the system when the software is opened.

## Job details

Job details will summarize the necessary information of a Job so that users can observe before selecting a Job to perform the process.

Note: there is an important information that is Job status, which indicates the status of a process.

## Buttons to manipulate Job

The Delete button will delete the selected Job

Button Next will select Job to move to the process execution page

## Job Classification

In the Job list, there is a color bar to categorize whether the Job was created for use with the printer or the Job created for use without the printer (Standalone).

# Chapter 8 – Job execution page

This is the page to start running the process of the selected Job

## Job details

This section displays overview information of the current active Job.

## Image display box

When running the process, a photo will be taken by the camera and displayed here

## Verification results

Display data and general information of the data including:

Code: data content read from camera

Processing time: processing time of data comparison

Status: status of comparison result

## Verification progress

This is where the evaluation results of the process are displayed, such as:

Percentage of the process

Total checked: total number of verified data

Passed: number of barcode data that was successfully verified

Failed: number of barcode data that was failed to be verified

## Statistics of POD data

This is where important data is aggregated when working with POD

Data sent: total data sent from R-Link to the printer, only exists in one session.

Data received: total data that the printer has responded to R-Link, only exists in one session.

Data printed: total data that the printer has responded to R-Link, exists and is kept for each Job

## Database View

Displays the data list and print status directly with the printing process. The Status column will display two statuses

Click on the Database or Data printed menu to display the Database View window

Waiting: data not yet printed

Printed: data already printed

Refresh button: refresh the list

Search button: search data by keyword

## Job logs

Where camera data and results are displayed along with verification results

Verification results can be

Valid: Valid verification result

Invalided: Invalid verification result

Duplicated: Duplicate verification results

Null: Verification result is empty

Unknown/Missed: Unverified or unknownOperation Buttons

Start button: start running the verification process

Stop button: stop the verification process

Camera trigger button: click to take a picture manually.

## Job Logs Detail

Filter : filter sorts data by results

Reprint: perform the process of reprinting the failed data

Recheck: perform the check process again with Barcode scanner

Refresh: refresh the data list

Search: search data by keyword

Statistics of verification results

- Total code of the job

- Number of printed code

- Number of verified code

- Number of valid code

- Number of failed code

# Chapter 9 – Users management

## Users Management

On the vertical menu bar, select the Account item to open the user management page

Click Manage

Here, the Account list section is where the list of current users on the software is displayed.

## Users management functions

Full name: the name you use to identify yourself, optional

Username: username to log in

Password: password to log in

Retype password: used to confirm the password a second time

Role: user role

Add user

Edit user info

Save user info

Remove user account

## User management operations

Add user

Steps to follow

Step 1: Enter user information

Step 2: Save user

Step 3: Confirm notification of successful user addition

Added users are updated on the list

Remove user

Steps to follow

Step 1: Select the user to delete

Step 2: Click the delete user button

Step 3: Confirm user deletion

User has been removed from the list

Edit user information

Only user roles can be changed

# Chapter 10 – Program history

Program history is a function that records the log of the main activities of the verification process. Currently supports logging of:

Login/Logout

Start/Stop

Log Filter

Filters include: Error, Warning, Info, Login, Logout, Start, Stop

Buttons for working with logs

Clear log: Clear all log

Refresh: Refresh log list

Export : Export log to file

## Log datetime filter

Select the time period to display relevant log histories

# Chapter 11 – Export reports for Job

## Export print data report

On the vertical menu bar, select the Data export item as shown below to export data.

A dialog box appears, select where to save, name the file and click the Save button.

## Export verification data report

On the vertical menu bar, select Export Result

A dialog box appears, select where to save, name the file and click the Save button.

# Chapter 12 – Another functions

## Return to Job selection page

On the vertical menu bar, select Job to return to the Job selection page interface.

## Status bar and other information

Below the Job execution interface is a status bar that shows the device connection status, process status, and other information.

(1) Icons show the connection status of devices such as: Camera, Printer, Scanner, PLC

(2) Process Status (Running, Processing, Stopped) |Software Version |Current Datetime

# Chapter 13 – Rynan printer settings

This chapter will cover how to install a printer using the Rynan R series printers.

## Check all printer settings function

R-Link has the function of checking the settings on the printer. You need to enable the Check all printer settings function in the Printer settings section to use it or you can install it yourself without this function.

For the printer to work with R-Link you need to enable the following main features:

POD (1)

Response POD Data (2)

Response POD Command (3)

Monitor (4)

In addition, you need to set the following rules:

Split character: Semicolon

Encoding: 65001 – Unicode (UTF-8)

## Notification of printer parameters not enabled

If the Check all printer settings function is enabled, if any settings are not satisfied, a notification dialog appears as shown below.

## Printer connection settings

The printer will use TCP/IP protocol to communicate with R-Link software so you need to install it to get

IP Address: can use Dynamic or Static IP function depending on usage needs, but must have a specific IP Address

Port: to work with POD, you need to set a specific port, you can customize it here, the default will be 2030

## Design a template

In order for R-Link to identify the template to print, you need to design a print template with a specific name. R-Link will find it in the list of print template names.

# Chapter 14 – Cognex Dataman camera settings

## Software required

Software to configure Dataman camera series : Dataman Setup Tool

Download free software on website:

https://support.cognex.com/en/downloads/dataman/software-firmware

## Connect to camera

Open the software with the icon as follows

Step 1: First connect the camera and computer via Ethernet Cabble.

Step 2: Predetermine the camera's network layer or use DHCP on the computer to detect.

Step 3: Select the found camera.

Camera setup interface on Dataman Setup Tool software

## Optimize Image

This function is used to configure the barcode image to be optimal for camera recognition.

Tune: automatically tunes the image to optimize data reading

You can also manually fine-tune it

## Identify barcode type

The camera supports identifying the type of barcode to work with, all can be selected, however to optimize performance, it is necessary to specifically identify the type of barcode to work with.

## Trigger type

To command the camera to take a photo, you need to configure the trigger type to use. Here, use single (external) to use an external sensor to trigger the camera to take a photo.

## Output settings

To let the camera receive output conditions from the R-Link software, the User Event function needs to be enabled.

Pulse with (ms): Adjust the output opening time on the camera, adjust it to match the error classification system.

Camera IP address

Go to the Communications menu to set up the camera's IP configuration.

- DHCP is a protocol used to automatically assign IP addresses to devices on a network. The IP address assigned can change over time (dynamic).

- A static IP address is manually assigned to a device and remains constant over time. (recommended)

# Chapter 15 – Cognex In-Sight Vision camera settings

Coming soon